Inhalt

Trace width calculation	2

Power sauna oven with 6KW	2

Results for 6KW	2

Power sauna oven width 9KW	3

Results for 9KW	3

# Trace width calculation

## Introduction

From the web:

- Tmax chosen = 80°C

## Power sauna oven with 6KW

Current per line:	6KW complete, 2KW per line
			Iline (L1, L2, L3) = P / U = 2000/230 = 8,7A 
			Chosen for calculation 9A

Trace width: 		3,0mm

Caclulator on: https://www.leiton.de/leiton-tools-leiterbahnerwaermung.html

### Results for 6KW

Delta T expected: + 28°C

Final trace temperature: 22°C room + 28°C heating = 50°C	=> OK

## Power sauna oven width 9KW

Current per line:	9KW complete, 3KW per line
	Iline (L1, L2, L3) = P / U = 3000/230 = 13,04A
Chosen for calculation 13A

Trace width: 		3,5mm

### Results for 9KW

Delta T expected: + 47°C

Final trace temperature: 22°C room + 28°C heating = 69°C	=> OK

Although I have an 6KW oven, the design should be also OK for 9KW.
If possible 3.5 mm traces should be used.

# Addendum

## Table for temperature rise depending on current and trace width